# Using CheD for data addition to SpecInfo databases

Written by Sergei V. Trepalin, trep@chemical-block.com

Last revised: 20 July 2007.

## How CheD can be used

CheD can be used wih SpecInfo only by export/import  of JCAMP files. No direct connection is possible!

CheD might be treated as local file-server, which supports chemical data-chemical structures, spectra, etc. CheD has tools for chemical structure drawing and peak assignment to atoms in chemical structure.

## Definition of multiplet

An NMR multiplet can be treated as a number of peaks, the distance between them is determined coupling constants (JJ). Usual way of NMR peak representation is PPM, usual way of JJ coupling constant representation is Hz. A nucleus can be coupled with several magnetically non-equivalent nucleus. In the case one has complex multiplet. CheD can manage multiplets, which arise during interaction of any number of nucleus.

A multiplet is characterized by its position (X), intensity (Y) and text description. User can edit all of the values. In the case user-define value will be used for these parameter. So, data imported from JCAMP file and stored as multiplets contain user-defined values and below default values cannot be implemented.

User-defined multiplet description is shown at spectrum image without text in brackets. So, if one will define description as D[8.5Hz]T[2.2Hz], at spectrum image one can see only DT. So, additional information should be inserted into brackets. Brackets should be balanced-otherwise at spectrum image one will see description, which will be started from ‘*’ and terminated also with ‘*’.

By default, parameters are calculated as:

Position is defined as sum of positions (PPM) of all peaks, forming multiplet, divided by the number of peaks

Intensity is defined as sum of intensities (relative units) of all peaks, forming multiplet, divided by the number of peaks

Description is filled with standard abbreviations for multiplets (S,D,T,Q, etc.). Additionally if no user-defined parameter is used in description and spectrometer frequency is known, JJ coupling constants are calculated and are added to description.

## Multiplets displaying in CheD

A special field – NMR multiplets was introduced to save data for NMR multiplets. It is defined in a database in usual way-see manual.

To display NMR multiplets field, execute Edit/View form, draw appropriate box (enough large size shuld be used, example 200*200 pixels) and select field with multiplets list from dropped-down combobox (item 1 at below picture).

It is recommended to draw Multiplet control, at the same tab, where chemical structure is displayed-in the case one can see assignment while selecting an item in multiplet list.

There are two possible ways for multiplets representations – as table with X,Y,Multiplicity data and as lines, drawn at spectrum image. First representation gives detailed information about atom assignment and JJ coupling constants, while representation on spectrum image will give distinct information, from what peaks multiplets were arisen. To change kind of data representation, use Picture combo box (item 3 at above picture). Both representation at single form are available-so one can see both multiplet list and spectrum image with multiplets at time.

CheD can handle a lot of  spectra with a lot of multiplicity fields in a single database. To resolve ambiguities, on what spectrum image given multiplet list should be displayed, select spectrum field for give multiplet list – item 2 at image above.

When one browse through CheD database content and Edit switch is settled to On, one can edit multiplets with button clicking at top right corner of Multiplet List control. In the case, multiplet edit form is arisen.

## Multiplet list edit form

The form is used to edit multiplets-their position, intensity, multiplicity description, assignment and JJ coupling constants.

Main purpose of this form is to combine peaks into multiplets. JJ coupling constant is  Moreover, defined multiplets might be combined also, to form complex multiplets any depth – like DD, DT, DTD, etc. JJ coupling constants are calculated automatically annd are displayed in the table.

Meaning of Undo, Redo, OK and Cancel commands are obvious. Commands Clear Assignment, Delete peaks or Split Multiplets, Combine peaks, also as assignment to atoms can be executed only if some multiplet list items are selected. To select items, one can use two ways:

- Click at multiplet descriptions in list box. Ctrl and Shift  keys might be used to select multiply items in list

- Click at multiplet description at Spectrum Image. One click will select an item, the second click will remove selection. To zoom spectrum image, drag with mouse on it while Ctrl key is holding down. To unzoom-double click at it. To scroll spectrum image, drag mouse on image.

Change observed frequency – set new value for observer frequency (MHz). It is required for JJ coupling constants calculations.

Clear Assignment – remove assignment to atoms for selected items.

Delete peaks or split multiplets – the command either splits multiplets into more simple multiplets (peaks) or deletes selected peaks. If one, for example select DD, this command will give two separate doublets. If one will select double, the command will give two singlets. Finaly, if a singlet will be selected, the command will delete it from multiplet list. Confirmation of this command is absent because of one can use Undo command easy.

Combine peaks is enabled only if many multiplets or peaks are selected in the listbox. Selected peaks will form multiplet, the JJ coupling constants and appropriate multiplet description will be calculated automatically if uniform multiplets will be selected. Example: if one will select two singlet, a dublet will be formed. If one will select three dublets, a triplet of dublets will be formed. But if one will select singlet and dublet, a multiplet with unknown JJ coupling value will be formed.

Add losted peak – enables ones to define again singlets, which were erroneously deleted by Delete Peaks command or which were not inserted in original JCAMP file, or which were not detected by peak peaking process. This is toggled button-when one will click on it, all command become unavailable. One should click at spectrum image in desirable places to add peaks. One should click at this button again to stop peak addition and continue manipulations with multiplets.

Read Peaks – erase all multiplet list content and read peaktable from declared in above ComboBox  field. All peaks are treated as singlets and might be combined in multiplets by Combine command. This command is useful for exact multiplet creation instead of saved in JCAMP file, because of JCAMP file contains not full multiplet creation- JJ coupling constants and peak positions are losted.

Atom assignment is made on structure image. One has to select a number of multiplets prior assignment. Click at atom to put assignment. Second clicking will remove assignment. Use Clear Assignment button to remove all assigned atoms.

## Import data from external JCAMP fils (instrumental or generated by third-party soft)

### Database creation

One has to created a database, which has to contain

Spectrum		PackedArray

PeakTable		Real,Array with size=1 (this field might be omitted if spectrum contains ##PEAKASSIGNMENT module)

Multiplets		NMRMultiplets

SpectrometerFrequncy	Real

Except these field, a database might contain a number of another fileds (CompoundName, CAS, etc). How to create/Edit/Delete fields one can find in CheD manual, which is located in the directory <ProgramSetupDir>/Manuals (by default <ProgramSetupDir> is C:/CHED)

After field addition one should change visible form to display data added. How to do this-read CheD manual.

One can use attached database Data1H as such predefined database.

### Import of external JCAMP files in CheD

Described below import process is implemented to those JCAMP files, which contains single NMR spectrum in a JCAMP files. To import a JCAMP file with multiply spectra (JCAMP-link file) one has to execute File/Import command and select JCAMP as filter.

One should put such files (with single spectra each) in separate directory. Then one should open above database (empty or which already contains data). Turn Edit Switch at database window to On and execute command Spectra/JCAMP IR or NMR spectrum/NMR from menu. Next window is arisen:

One should select a path to directory at top left box and then click at JCAMP file in file listbox (next right from path box). Then one has to create field matching in two right listboxex-so to define which field in JCAMP file corresponds to which field in database. To match pair of field, click at white checkbox from right listbox with database fields and move mouse when left button is holding down to a checkbox near control with JCAMP file fields and then release button (Drag and Drop). Fields assignments are shown with straight lines To clear assignment, click at checkbox with line, move mouse outside it and release button at any place.

After assignment will have been done, one can execute commands from bottom rights buttons-Edit,Append and All.

Edit – substitute values for current records in CheD database with those values from assigned JCAMP file fields.

Append – append data from JCAMP files as new record in CheD database.

All – this button is used when a number of uniform (with identical data) files are inserted into selected directory. After execution of this command, content of first JCAMP file is appended as new record to CheD database. Then next JCAMP file is selected automatically and procedure is repeated again and so on until last file. So, one has batch data addition. It is important, that all JCAMP files have identical data field names – for example, a field, which contains IDNUMBER has label ##COMPOUNDID in all files. Usually files from single source (instrument or soft) satisfy to this requirement.

Multiplets  - displays multiplets edition form. But it is not recommended to edit multiplets during data import. It would be better to save peaks in database and then edit multiplets after chemical structure will be added.

## Structure addition and peak assignment

Please read Stred manual in the directory C:/CHED/MANUALS for explanation, how to draw chemical structure

Please read CheD manual n the directory C:/CHED/MANUALS for explanation, how to create peak assignment to atoms

## Export of CheD database into JCAMP file for SpecInfo

Open database with spectral data and then execute command File/Export from menu. In the standard File Save As dialog select JCAMP-Link file in Save as Type combobox. Do not remove checkmark “Create separate Link block for each record in database”! Select file and path and click OK button.

Next dialog is arisen then:

At first, one has to define a block to be exported. To do this one should click at Spectrum field at left list box (Database field) and move mouse while left button is holding down and release button at ##XYDATA line in NMR spectrum block (drag and drop). Then one can assign another fields from database or fixed values to blocks CHEMICAL STRUCTURE and NMR spectrum. See CheD manual in the C:\CHED\MANUALS for detailes.

Multiplet list field can be matched with ##PEAK ASSIGNMENT data type in JCAMP file and exported there.

Once assigned database model can be saved in file with Save button. Instead of next assignment while database will be opened next time, one can open this model with Load button click

Export is started with OK button click